ECDIS – Check Cards

During Passage the ECDIS shall be set up as follows, unless specifically stated by the master in the passage plan. NAVISAILOR will have these three modes as pre-set configurations. The alarms and displays are the minimum.

PILOTAGE (within 2nm safety contour)

SHALLOW - 2m

SAFTEY CONTOUR - 5m

SAFETY DEPTH - 10m

DEEP CONTOUR - 20m

SPOT SOUNDSINGS - 50m

SYMBOLS - PAPER CHART

CHART ZOOM - 1:1

CHART PRIORITY - ENC

CHART AUTO LOAD - ON

DEPTH SHADES - FOUR SHADES

CORRECT ROUTE – LOADED

RADAR IMAGE OVERLAY – ON

SHIP VECTORS – 3mins

MONITORING ON – BEST SCALE

SAFETY FRAME - ON

HARBOUR = 3mins & 0.5cb

PILOTAGE = 5mins & 1cb

DISPLAY CATEGORY - CUSTOM &

CABLES / PIPLINES

SEABED

SHALLOW PATTERN

COASTAL (within 12nm safety contour)

SHALLOW - 5m

SAFTEY CONTOUR - 15m

SAFETY DEPTH - 20m

DEEP CONTOUR - 30m

SPOT SOUNDSINGS - 50m

SYMBOLS - PAPER CHART

CHART ZOOM - 1:1

CHART PRIORITY - ENC

CHART AUTO LOAD - ON

DEPTH SHADES - FOUR SHADES

CORRECT ROUTE – LOADED

MONITORING ON – BEST SCALE

SHIP VECTORS – 6mins

SAFETY FRAME - ON

LOOKAHEAD = 10mins & 0.5Nm

DISPLAY CATEGORY - CUSTOM &

NAMES

FULL LIGHT LINES

SCAMIN

OPEN (outside 12nm safety contour)

SHALLOW - 10m

SAFTEY CONTOUR - 30m

SAFETY DEPTH - 30m

DEEP CONTOUR - 50m

SPOT SOUNDSINGS - 100m

SYMBOLS - PAPER CHART

CHART ZOOM – RADAR RANGE

CHART PRIORITY - ENC

CHART AUTO LOAD - ON

DEPTH SHADES - FOUR SHADES

CORRECT ROUTE – LOADED

MONITORING ON – BEST SCALE

SHIP VECTORS – 12mins

SAFETY FRAME - ON

LOOKAHEAD = 15mins & 2Nm

DISPLAY CATEGORY  CUSTOM &

SCAMIN

NATIONAL NAMES

FULL LIGHT LINES

Pre-sets in all conditions: Chart motion to Relative Motion, North up.  Off set the vessel to provide suitable look-ahead.

Display settings may be changed by the OOW as required, subject to the situation. A better display of the chart data and required information may be possible, such as during anchoring operations or pilotage. Settings should be reverted to the above after, and before handing over the watch. Relieving officers shall confirm the above and other settings before taking command of the watch.