Navigation in Restricted Visibility

| ACTION |  |
| --- | --- |
| Inform Master |  |
| Plot Position & cross reference using Navigation aids |  |
| Adjust speed as necessary. (COLREGS Rule 6: Safe Speed) |  |
| Radar – Set range scale as needed. |  |
| Select appropriate fog signal. (COLREGS Rule 35: Sound signals in areas of Restricted Visibility) |  |
| Check Navigation lights on at full brilliance |  |
| Engines to be immediately available |  |
| Start second steering pump |  |
| Post extra look-outs |  |
| Prepare bridge (open doors etc ) and alert bridge to listen for sound signals from other vessels. |  |
| Inform on duty engineer |  |
| Closely monitor VHF |  |
| Monitor depth sounder |  |
| Watertight doors closed – Command discretion |  |
| Entry in Deck Logbook |  |
| Consider appropriate anchor plan if necessary |  |