Before taking over the Watch and signing the Log the following checks must be completed.

| By the outgoing OOW |  |
| --- | --- |
| Are all members of the relieving watch fully capable of performing their duties? |  |
| Do not complete the handover until any manoeuvre to avoid shipping or alter course in accordance with the passage plan is complete |  |

By the incoming OOW

Standing and Night Orders - Read, sign and understand nights orders

Navigation situation - Plot a fix on the paper chart / ECDIS using means other than the primary GNSS or feed to the ECDIS - Confirm vessel is in the correct location according to the passage plan and will meet the destination ETA. If off track, confirm why and when due to regain - Are all appropriate charts / cells available for the forthcoming watch - Are the correct user profile settings in place on the ECDIS - If available – turn on the AIO layer and check for any relevant notices - Confirm the datum in use is appropriate for the area of passage / operations - Check the gyro heading is correct

Operations - Confirm any activities planned for the forthcoming watch and next 24 hours

Command - Where is the Captain (or C/O if holding temporary command), any particular means to call him

Weather - Step outside to check, cloud cover, rain, visibility, swell, sea state - Check stern arcs - What is the forecast

Internal - Any equipment faults affecting your passage or safe operation - Any activities planned during the watch to be aware of - Where are the tenders (if launched), helicopter (if carried). What movements are planned - Are navigation lights on - Any sound signals (due to restricted visibility) - State of the engines and other propulsion and steering

Safety - Is anyone working aloft, over the side or in a confined space? Is the permit to work complete - Are all guests and crew in board - In the upperdeck out of bounds

Shipping - Starting from the bow and working clockwise, identify all contacts visually and on radar confirming: - Their bearing drift - CPA - What they are - Whether contact has been made with them - Intentions

THEN RUN THROUGH THE STATE OF THE ENGINES, COURSE STEERING AND COMPLETE THE HANDOVER FORMALLY ANNOUNCING WHO HAS THE CONN / SHIP

| Has a record of the change of OOW been recorded in the Ship`s Log? (signed by the outgoing OOW to confirm an accurate reflection of their watch) |  |
| --- | --- |
| Notes: |  |