|  | Areas Covered | Comments | Date Completed |
| --- | --- | --- | --- |
| 1 | Initial Preparation |  |  |
| 1.1 | Locate ECDIS Procedures in the SMS and Masters Standing Orders. Explain key points |  |  |
| 1.2 | Locate any vessel specific ECDIS checklists |  |  |
| 1.3 | Explain what other systems feed into the ECDIS including the ability to find radar, AIS, GPS, STW, SOG, COG, Gyro including secondary feeds |  |  |
| 1.4 | Locate the ECDIS Manufacturer User Manual |  |  |
| 1.5 | Set up the primary and secondary fixing GNSS correctly and confirm datum output |  |  |
| 2 | Basic Operation |  |  |
| 2.1 | Switch the ECDIS on / off |  |  |
| 2.2 | If necessary, having turned the ECDIS on, start the ECDIS software |  |  |
| 2.3 | Demonstrate day / dusk / night viewing |  |  |
| 2.4 | Switch between traditional and simplified symbology, plain and complex line styles |  |  |
| 2.5 | Establish which ENC objects will alarm and which warn. Demonstrate how to upgrade indications to alarms |  |  |
| 2.6 | Show how to set safety contour and safety depth explaining the difference between each |  |  |
| 2.7 | Understand all relevant overlays and explain the basic functions, including manual layers with pop-up warnings |  |  |
| 2.8 | Understand how to check the correct vessel settings are loaded including offsets for sensors |  |  |
| 2.9 | Show that the compass reading on ECDIS matches the reading on the physical compass (gyro and magnetic) |  |  |
| 3. | Charts |  |  |
| 3.1 | Identify if the vessel is on pre-licence or Pay-As-You-Sail (PAYS). Explain the difference |  |  |
| 3.2 | Demonstrate how to select the required charts / amend the active folio, then load the permits within the ECDIS |  |  |
| 3.3 | Show how to load the ENC’s to the ECDIS |  |  |
| 3.4 | Demonstrate how to remove expired ENC’s from the ECDIS |  |  |
| 3.5 | Demonstrate how to check the update status of ENCs within the ECDIS and request weekly updates |  |  |
| 3.6 | Display and explain the meaning behind CATZOCs |  |  |
| 3.7 | Determine how to insert manual updates and explain why they would be used |  |  |
| 3.8 | If available, demonstrate how to switch between Vector and Raster charts for display |  |  |
| 3.9 | Understand how to ensure the chart motion, chart orientation, screen layout, colour, palette and additional ENC settings are configured correctly |  |  |
| 4. | Navigational Tools and Functions |  |  |
| 4.1 | Determine how to access the Presentation Library edition |  |  |
| 4.2 | Complete a pick report and explain the detail within it |  |  |
| 4.3 | Determine how to change ENC display settings for base, standard and custom |  |  |
| 4.4 | If the feature is available, show how to create a user profile of display settings and how to switch to show these |  |  |
| 4.5 | Demonstrate how to measure range and bearings using EBL’s and VRM’s |  |  |
| 4.6 | If available, demonstrate how to set up PI’s |  |  |
| 4.7 | Understand navigation alarms, how they are configured, including safety frame / anti-grounding cone |  |  |
| 4.8 | Determine how to view, add, edit and delete Mariners notes |  |  |
| 4.9 | Understand how to use any other navigation tools available and configure correctly (if applicable) |  |  |
| 5. | Route Planning |  |  |
| 5.1 | Discuss the points for consideration prior to route planning |  |  |
| 5.2 | Set the display and relevant layers for route planning taking into account considerations for dangers |  |  |
| 5.3 | Plan, load and edit routes with a suitable Cross Track Distance (XTD). Explain the purpose of the XTD |  |  |
| 5.4 | Set up the display for pilotage, coastal and then open waters explaining what differences have been made and why |  |  |
| 5.5 | Determine how to use route verification and checking for dangers along the route |  |  |
| 5.6 | If possible, lock an approved route to prevent unauthorised amendments |  |  |
| 6 | Route Monitoring |  |  |
| 6.1 | Load a pre-planned route |  |  |
| 6.2 | Determine how to display radar and AIS overlays if available |  |  |
| 6.3 | If radar and AIS targets can be displayed on the ECDIS determine what target vector modes are available and how to switch and differentiate between them |  |  |
| 6.4 | Demonstrate how to input Lines Of Position (LOP) and radar ranges to form an estimated position |  |  |
| 6.5 | Determine how to switch to dead reckoning mode and to identify when the ECDIS is in dead reckoning mode |  |  |

I can confirm the above ECDIS familiarisation training has been completed and understood and that the individual tested holds a Flag State approved IMO Generic ECDIS certificate:

Trainee Name ……………………………………………………………………. 	Signature …………………………………………………………

Examiner Name ………………………………………………………………… 	Signature …………………………………………………………

Date …………………………………………………………………………………..